# Bitcoin Switch Firmware – ESP32 + BTCPay

Oct 14, 2026 · @Bongani

## Get the code

This ESP32 sketch turns paid Lightning invoices on BTCPay Server into arcade credits by clicking a relay on the coin input.

Get the code: copy the whole sketch from Full code at the end of this page and paste it into a new Arduino sketch named bitcoin_switch.

The settings at the top hold placeholders only. Don't share a copy that has your Wi-Fi password or API key filled in.

## What it does

The ESP32 asks BTCPay for paid invoices every 4 seconds and turns each new one into coin clicks.

payment to coin · 6 parts

The ESP32 saves each invoice before the relay clicks, so a reboot can never pay the same invoice out twice.

## What you need

One ESP32, one relay and a BTCPay store with Lightning are all the switch needs.

| Item | Details |
| --- | --- |
| ESP32 dev board | Any ESP32 DevKit. Select Node32s or ESP32 Dev Module in Arduino IDE. |
| Relay module | 1 channel, 5 V. Its contact stands in for the coin switch. |
| USB cable | Must carry data. Connects the ESP32 to the Raspberry Pi. |
| Arduino IDE | 1.8.19 (2.x works too) |
| ESP32 board package | ESP32 Arduino core 2.0.14, from Boards Manager |
| Library | ArduinoJson 7.x by Benoit Blanchon, from Library Manager |
| BTCPay Server | A store with Lightning enabled and an API key |

## BTCPay setup

The ESP32 only reads invoices, so its API key needs just one permission: View invoices.

- Create a store just for the arcade, with Lightning enabled. Turn off on-chain payments, because confirmations take too long for a game.

- In that store, add a Point of Sale app with one item, for example "1 Credit" at R5.00. Its link (https://your-server/apps/<APP_ID>/pos) is what players scan.

- Go to Account → API Keys → Generate Key, tick only View invoices for that store, and copy the key.

- Copy the Store ID from Store Settings → General.

If the store also sells other things, set ORDER_ID_FILTER to pos-app_<APP_ID> so only the arcade's Point of Sale counts.

## Flash it

Fill in the settings at the top of the sketch, then upload it like any other ESP32 sketch.

| Setting | What to put | Example |
| --- | --- | --- |
| WIFI_SSID, WIFI_PASSWORD | Your Wi-Fi name and password |  |
| BTCPAY_URL | Your server address, with no slash at the end | https://btcpay.example.com |
| STORE_ID | From Store Settings → General |  |
| API_KEY | The View invoices key |  |
| CREDIT_PRICE | Price of one credit, in the store currency | 5.00 |
| RELAY_ACTIVE_LOW | true if your relay switches on when IN is LOW | false |

- Sketch → Include Library → Manage Libraries → install ArduinoJson version 7.x.

- Tools → Board → Node32s (or ESP32 Dev Module), then pick the COM port.

- Click Upload.

If the upload stops with "Failed to connect… waiting for packet header", hold the BOOT button while it says "Connecting…". If that doesn't fix it, set Tools → Upload Speed to 115200 and use a USB cable that carries data.

## Wiring

Each relay click is one coin, so the relay contact goes in parallel with the arcade's coin switch.

| From | To | Note |
| --- | --- | --- |
| ESP32 GPIO 13 | Relay IN | To use another pin, change RELAY_PIN. Avoid GPIO 0, 2, 12 and 15. |
| ESP32 VIN (5 V) | Relay VCC |  |
| ESP32 GND | Relay GND |  |
| Relay COM and NO | The arcade's two coin-input wires | In parallel with the coin button or coin acceptor |
| ESP32 USB | Raspberry Pi USB | Powers the ESP32 and sends credit messages to the display |

## Test and troubleshoot

Open the Serial Monitor at 115200 baud. HB OK every 5 seconds means the switch is online and watching BTCPay.

| Message | Meaning |
| --- | --- |
| WIFI OK <ip> | Joined Wi-Fi |
| HB OK, HB NOWIFI, HB NOTIME, HB APIERR | Heartbeat every 5 s: online, no Wi-Fi, clock not synced yet, can't reach BTCPay |
| CREDIT 2 <invoiceId> | A payment for 2 credits. The relay clicks twice. |
| INFO first start: skipped … | On its very first start, the switch ignores invoices that were already paid |
| ERR BTCPay replied 401 (or 403, 404) | The Store ID or the API key permission is wrong |

Type TEST and press Enter: the relay clicks once and CREDIT 1 TEST appears.

| Problem | Fix |
| --- | --- |
| Relay stays on at boot, or clicks the wrong way round | Set RELAY_ACTIVE_LOW = true |
| Game misses some coins | Set PULSE_MS = 150 and PULSE_GAP_MS = 400 |
| HB APIERR keeps showing | Read the ERR line printed just before it |

## Full code

Copy everything in the box into a new Arduino sketch named bitcoin_switch, then fill in the settings section.

/*
  Bitcoin Ekasi Arcade - Bitcoin switch firmware (ESP32 + BTCPay Server)
  ------------------------------------------------------------------------
  Watches a BTCPay Server store for paid ("Settled") invoices. For every new
  paid invoice it:
    1. works out how many credits were bought (paid amount / CREDIT_PRICE)
    2. remembers the invoice in flash, so it can never be paid out twice,
       even after a reboot or power cut
    3. pulses the relay once per credit (the relay closes the arcade's coin input)
    4. tells the Raspberry Pi over USB serial:  CREDIT <n> <invoiceId>

  Messages sent to the Pi (115200 baud, one per line):
    BOOT ...           firmware started
    WIFI OK <ip>       connected to Wi-Fi
    CREDIT <n> <id>    n credits were paid for
    HB <state>         heartbeat every 5 s; state = OK | NOWIFI | NOTIME | APIERR
    INFO / ERR ...     extra detail, handy in the Arduino Serial Monitor
  Commands it accepts (from the Pi, or typed into the Serial Monitor):
    TEST               pulse the relay once and send "CREDIT 1 TEST" (wiring check)

  Board:    Node32s (or "ESP32 Dev Module"), ESP32 Arduino core 2.0.x
  Library:  ArduinoJson 7.x by Benoit Blanchon (Sketch > Include Library > Manage Libraries)
            Everything else (WiFi, HTTPClient, Preferences) comes with the ESP32 core.
            The old LNbits libraries (WebSockets, PageBuilder) are NOT needed.
*/

#include <WiFi.h>
#include <WiFiClientSecure.h>
#include <HTTPClient.h>
#include <Preferences.h>
#include <ArduinoJson.h>
#include <time.h>

#if ARDUINOJSON_VERSION_MAJOR < 7
#error "This sketch needs ArduinoJson 7.x - update it in Sketch > Include Library > Manage Libraries"
#endif

// ================================ SETTINGS ===================================

// Wi-Fi
const char* WIFI_SSID     = "YOUR_WIFI_NAME";
const char* WIFI_PASSWORD = "YOUR_WIFI_PASSWORD";

// BTCPay Server
const char* BTCPAY_URL = "https://YOUR-BTCPAY-SERVER";  // no slash at the end
const char* STORE_ID   = "YOUR_STORE_ID";               // Store Settings > General > Store ID
const char* API_KEY    = "YOUR_API_KEY";                // Account > API Keys, permission "View invoices" only

// Only count invoices from one Point of Sale app (use this if the store sells other things).
// Format: "pos-app_" + the app ID from the POS link (.../apps/<APP_ID>/pos).
// Leave "" to count every paid invoice in the store.
const char* ORDER_ID_FILTER = "";

// Credits
const float CREDIT_PRICE = 5.00;           // price of ONE credit in the store currency (e.g. R5.00)
                                           // 0 = every paid invoice gives exactly 1 credit
const int   MAX_CREDITS_PER_PAYMENT = 20;

// Relay
const int  RELAY_PIN        = 13;     // GPIO -> relay IN. Avoid 0, 2, 12, 15 (boot pins)
const bool RELAY_ACTIVE_LOW = false;  // set true if your relay clicks ON when IN is LOW
const int  PULSE_MS         = 100;    // how long the "coin" contact stays closed
const int  PULSE_GAP_MS     = 300;    // pause between coins

const int  LED_PIN = 2;               // onboard LED: on = Wi-Fi and BTCPay OK. -1 to disable

// Polling
const unsigned long POLL_INTERVAL_MS = 4000;
const long LOOKBACK_SECONDS = 3600;   // check invoices created in the last hour

// HTTPS: nullptr = encrypted, but the server certificate is not checked.
// Paste your server's root CA certificate (PEM) here to have it checked.
const char* ROOT_CA = nullptr;

// =============================================================================

const int RELAY_ON  = RELAY_ACTIVE_LOW ? LOW : HIGH;
const int RELAY_OFF = RELAY_ACTIVE_LOW ? HIGH : LOW;

// Invoices already paid out, stored as 64-bit hashes of the invoice ID.
// Must hold more entries than the store could settle within LOOKBACK_SECONDS.
const int SEEN_SLOTS = 200;
uint64_t seen[SEEN_SLOTS];
uint16_t seenNext = 0;
bool baselineDone = false;   // first boot: existing invoices are skipped, not credited
Preferences prefs;

enum State { ST_NOWIFI, ST_NOTIME, ST_APIERR, ST_OK };
State state = ST_NOWIFI;
bool lastPollOk = false;
bool wifiWasUp = false;
unsigned long lastPoll = 0, lastHeartbeat = 0, wifiLostAt = 0;
String serialLine;

// ------------------------------------------------------------- remembering --

uint64_t idHash(const char* s) {          // FNV-1a
  uint64_t h = 1469598103934665603ULL;
  while (*s) { h ^= (uint8_t)*s++; h *= 1099511628211ULL; }
  return h ? h : 1;                       // 0 marks an empty slot
}

bool isSeen(const char* id) {
  uint64_t h = idHash(id);
  for (int i = 0; i < SEEN_SLOTS; i++) if (seen[i] == h) return true;
  return false;
}

void saveSeen() {
  size_t a = prefs.putBytes("seen", seen, sizeof(seen));
  prefs.putUShort("next", seenNext);
  if (a != sizeof(seen)) Serial.println("ERR could not save paid-invoice list to flash");
}

void rememberInvoice(const char* id, bool save) {
  seen[seenNext] = idHash(id);
  seenNext = (seenNext + 1) % SEEN_SLOTS;
  if (save) saveSeen();
}

void loadSeen() {
  memset(seen, 0, sizeof(seen));
  prefs.begin("arcade", false);
  if (prefs.getBytesLength("seen") == sizeof(seen)) {
    prefs.getBytes("seen", seen, sizeof(seen));
    seenNext = prefs.getUShort("next", 0) % SEEN_SLOTS;
    baselineDone = prefs.getBool("baseline", false);
  } else {
    baselineDone = false;   // nothing stored yet (or SEEN_SLOTS changed): start fresh
  }
}

// ------------------------------------------------------------------ relay --

void pulseRelay(int times) {
  for (int i = 0; i < times; i++) {
    digitalWrite(RELAY_PIN, RELAY_ON);
    delay(PULSE_MS);
    digitalWrite(RELAY_PIN, RELAY_OFF);
    delay(PULSE_GAP_MS);
  }
}

// ----------------------------------------------------------------- BTCPay --

float jsonNumber(JsonVariant v) {         // BTCPay sends amounts as strings, e.g. "5.00"
  if (v.is<const char*>()) return atof(v.as<const char*>());
  return v.as<float>();
}

int creditsFor(JsonObject inv) {
  if (CREDIT_PRICE <= 0) return 1;
  float amount = jsonNumber(inv["amount"]);
  float paid   = jsonNumber(inv["paidAmount"]);
  float value  = paid > amount ? paid : amount;
  int credits  = (int)floor(value / CREDIT_PRICE + 0.05f);  // 5% slack for rounding
  if (credits < 1) credits = 1;           // it is Settled, so it was paid in full
  if (credits > MAX_CREDITS_PER_PAYMENT) credits = MAX_CREDITS_PER_PAYMENT;
  return credits;
}

// Returns true if BTCPay answered properly.
bool pollBtcpay() {
  time_t now = time(nullptr);
  String url = String(BTCPAY_URL) + "/api/v1/stores/" + STORE_ID +
               "/invoices?status=Settled&take=100&startDate=" + String((long)(now - LOOKBACK_SECONDS));
  if (ORDER_ID_FILTER[0]) { url += "&orderId="; url += ORDER_ID_FILTER; }

  WiFiClientSecure client;
  if (ROOT_CA) client.setCACert(ROOT_CA); else client.setInsecure();

  HTTPClient http;
  http.useHTTP10(true);                   // no chunked replies, so the JSON can be streamed
  http.setTimeout(15000);
  if (!http.begin(client, url)) { Serial.println("ERR check BTCPAY_URL"); return false; }
  http.addHeader("Authorization", String("token ") + API_KEY);
  http.addHeader("Accept", "application/json");

  int code = http.GET();
  if (code != 200) {
    Serial.printf("ERR BTCPay replied %d%s\n", code,
                  (code == 401 || code == 403 || code == 404) ? " - check STORE_ID and API_KEY permissions" : "");
    http.end();
    return false;
  }

  // Keep only the fields we need, so a long invoice list fits in memory.
  JsonDocument filter;
  filter[0]["id"] = true;
  filter[0]["amount"] = true;
  filter[0]["paidAmount"] = true;

  JsonDocument doc;
  DeserializationError err = deserializeJson(doc, http.getStream(), DeserializationOption::Filter(filter));
  http.end();
  if (err) { Serial.printf("ERR reading BTCPay reply: %s\n", err.c_str()); return false; }

  JsonArray invoices = doc.as<JsonArray>();
  if (invoices.isNull()) { Serial.println("ERR unexpected BTCPay reply"); return false; }

  if (!baselineDone) {
    // Very first run: mark what is already paid as done, so old invoices don't pay out.
    for (JsonObject inv : invoices) {
      const char* id = inv["id"] | "";
      if (id[0] && !isSeen(id)) rememberInvoice(id, false);
    }
    saveSeen();
    prefs.putBool("baseline", true);
    baselineDone = true;
    Serial.printf("INFO first start: skipped %d invoices that were already paid\n", (int)invoices.size());
    return true;
  }

  // BTCPay lists newest first, so walk backwards to pay out oldest first.
  for (int i = (int)invoices.size() - 1; i >= 0; i--) {
    JsonObject inv = invoices[i];
    const char* id = inv["id"] | "";
    if (!id[0] || isSeen(id)) continue;

    int credits = creditsFor(inv);
    // Save BEFORE clicking the relay: if the relay causes a brown-out and the
    // ESP32 reboots, this invoice is already marked and can't pay out twice.
    rememberInvoice(id, true);
    Serial.printf("CREDIT %d %s\n", credits, id);
    pulseRelay(credits);
  }
  return true;
}

// ------------------------------------------------------------ connections --

bool timeIsSet() { return time(nullptr) > 1700000000; }   // after Nov 2023 = NTP has synced

void handleWifi() {
  bool up = WiFi.status() == WL_CONNECTED;
  if (up && !wifiWasUp) {
    Serial.printf("WIFI OK %s\n", WiFi.localIP().toString().c_str());
  } else if (!up && wifiWasUp) {
    Serial.println("WIFI DOWN");
    wifiLostAt = millis();
  } else if (!up && millis() - wifiLostAt > 30000) {
    WiFi.disconnect();                    // auto-reconnect gave up: start again
    WiFi.begin(WIFI_SSID, WIFI_PASSWORD);
    wifiLostAt = millis();
  }
  wifiWasUp = up;
}

void updateState() {
  if (WiFi.status() != WL_CONNECTED) state = ST_NOWIFI;
  else if (!timeIsSet())             state = ST_NOTIME;
  else                               state = lastPollOk ? ST_OK : ST_APIERR;
  if (LED_PIN >= 0) digitalWrite(LED_PIN, state == ST_OK ? HIGH : LOW);
}

const char* stateName() {
  switch (state) {
    case ST_OK:     return "OK";
    case ST_NOTIME: return "NOTIME";
    case ST_APIERR: return "APIERR";
    default:        return "NOWIFI";
  }
}

void readSerialCommands() {
  while (Serial.available()) {
    char c = Serial.read();
    if (c == '\n' || c == '\r') {
      serialLine.trim();
      if (serialLine.equalsIgnoreCase("TEST")) {
        Serial.println("CREDIT 1 TEST");
        pulseRelay(1);
      }
      serialLine = "";
    } else if (serialLine.length() < 32) {
      serialLine += c;
    }
  }
}

// ------------------------------------------------------------------- main --

void setup() {
  pinMode(RELAY_PIN, OUTPUT);
  digitalWrite(RELAY_PIN, RELAY_OFF);
  if (LED_PIN >= 0) pinMode(LED_PIN, OUTPUT);

  Serial.begin(115200);
  delay(300);
  Serial.println();
  Serial.println("BOOT bitcoin_switch 1.0 (BTCPay)");

  loadSeen();

  WiFi.mode(WIFI_STA);
  WiFi.setAutoReconnect(true);
  WiFi.begin(WIFI_SSID, WIFI_PASSWORD);
  wifiLostAt = millis();

  configTime(0, 0, "pool.ntp.org", "time.google.com", "time.cloudflare.com");
}

void loop() {
  readSerialCommands();
  handleWifi();

  if (millis() - lastPoll >= POLL_INTERVAL_MS) {
    lastPoll = millis();
    if (WiFi.status() == WL_CONNECTED && timeIsSet()) lastPollOk = pollBtcpay();
    else lastPollOk = false;
  }

  updateState();

  if (millis() - lastHeartbeat >= 5000) {
    lastHeartbeat = millis();
    Serial.printf("HB %s\n", stateName());
  }

  delay(10);
}